# Main Screens Commands

## Preinitialize Events

//Hide Mode Buttons

vis bLSB,0

vis bUSB,0

vis bCWL,0

vis bCWU,0

//Hide IFS

vis hIFS,qIFS.picc //found bug by VE1BWV

//Hide Text Line for compatiable with Character LCD

vis t0,0

vis t1,0

vis nCWNum,0

vis tCWText,0

vis bAction,0

//Added Version 1.1

vis bStep1,0

vis bStep2,0

vis bStep3,0

vis bStep4,0

vis bStep5,0

pm.vv.val=-1

pm.vg.val=0

pm.sp.txt="J"

pm.sb.txt="^"

vis tDecode1,0

vDecodeVisible.val=0

if(nQuickMenu.val==1)

{

//CW Menu

qKey.picc=1

vis nQWPM,1

vis tQKeyType,1

vis b0,1

vis b1,1

vis b2,1

vis hATT,0

vis hIFS,0

vis nATT,0

vis nIFS,0

}else

{

//Normal

qKey.picc=0

vis nQWPM,0

vis tQKeyType,0

vis b0,0

vis b1,0

vis b2,0

vis hATT,1

vis hIFS,qIFS.picc

vis nATT,1

vis nIFS,1

}

## TOUCH PRESS EVENTS

if(dim==0)

{

dim=dims

}

## ***TM1****

Events

if(pm.sp.txt!="J")

{

if(qInfo.picc==0)

{

qInfo.picc=1

fill 5,365,110,90,7

}

if(vDecodeVisible.val==1)

{

vis tDecode1,0

vDecodeVisible.val=0

}

sNewMax.txt=""

sys2=0

sys1=0

nTemp0.val=0

for(sys0=0;sys0<62;sys0++)

{

substr pm.sp.txt,sTemp0.txt,sys0*2,2

covx sTemp0.txt,sys2,2,2

//sys2/=2

if(sys1<sys2)

{

sys1=sys2

nTemp0.val=sys0

}

if(sys2<0)

{

sys2=0

}

if(sys2>100)

{

sys2=100   //was 90

}

substr sMax.txt,sTemp1.txt,sys0*2,2

covx sTemp1.txt,nTemp1.val,2,2

if(nTemp1.val<sys2)

{

nTemp1.val=sys2

}else

{

nTemp1.val*=9

nTemp3.val=sys2*1

nTemp1.val+=nTemp3.val

nTemp1.val/=10

}

covx nTemp1.val,sTemp1.txt,2,2

substr sTemp1.txt,sTemp1.txt,2,2

sNewMax.txt+=sTemp1.txt

if(sys0<18)

{

if(sys2>92) //was 82

{

sys2=92

}

if(nTemp1.val>92)  //was 82

{

nTemp1.val=92

}

fill sys0*4+5,475-nTemp1.val,4,nTemp1.val,1342	//1500+sys1*2046

fill sys0*4+5,475-sys2,4,sys2,57343	//1500+sys1*2046

fill sys0*4+5,383,4,92-nTemp1.val,7		//1500+sys1*2046

}else

{

fill sys0*4+5,475-nTemp1.val,4,nTemp1.val,1342	//1500+sys1*2046

fill sys0*4+5,475-sys2,4,sys2,57343	//1500+sys1*2046

fill sys0*4+5,365,4,100-nTemp1.val,7		//1500+sys1*2046

}

//if(sys1>nADCDisVal.val)					//maximum adc dccharget

//{

//  sys2=sys1-nADCDisVal.val

//}

//sys2=sys1*90	//80%

//sys2/=100

}

sMax.txt=sNewMax.txt

if(sys1>50)

{

sys1=63488

}else if(sys1>40)

{

sys1=64528

}else if(sys1>30)

{

sys1=58584

}else if(sys1>20)

{

sys1=39566

}else if(sys1>10)

{

sys1=33549

}else

{

sys1=31727

}

nTemp0.val=nTemp0.val*47

nTemp0.val=nTemp0.val+30

cov nTemp0.val,sTemp0.txt,0

xstr 20,362,50,20,2,sys1,7,0,1,1,sTemp0.txt

//xstr 5,350,90,18,0,WHITE,10565,0,1,1,sTemp0.txt

//xstr 5,360,37,18,0,sys1,7,2,1,1,sTemp0.txt

pm.sp.txt="J"

}

if(pm.sb.txt!="^")

{

if(vDecodeVisible.val==0)

{

vis tDecode1,1

vDecodeVisible.val=1

}

strlen tDecode1.txt,sys0

if(sys0>23)

{

substr tDecode1.txt,tDecode1.txt,1,sys0-1

}

tDecode1.txt=tDecode1.txt+pm.sb.txt

pm.sb.txt="^"

}

## ***TIMER 2***

//If Graphic mode

if(nDispStatus.val==0)

{

add 17,0,jSMeter.val/3

}

sys2=pm.vq.val&0x07

if(sys2>0)

{

//check Now condition

//if(dim==0)

if(nSaverCount.val==-10)				//Screen Saver Mode

{

//Screen Saver Mode

if(nLastFreq.val!=nSaverFreq.val)

{

dim=dims

nSaverCount.val=0

}

}else if(nSaverCount.val!=-20)	//Screen off by remote control -> disabled screen saver, //always backlight off

{

//Running Mode

if(nLastFreq.val!=nSaverFreq.val)

{

nSaverFreq.val=nLastFreq.val

nSaverCount.val=0

}else

{

nSaverCount.val++

if(sys2==1)

{

if(nSaverCount.val>=180)	//3 Min

{

dim=0

nSaverCount.val=-10

}

}else if(sys2==2)

{

if(nSaverCount.val>=600)		//10 Min

{

dim=0

nSaverCount.val=-10

}

}else if(sys2==3)

{

if(nSaverCount.val>=1200)		//20 Min

{

dim=0

nSaverCount.val=-10

}

}

}

}

}

if(pm.vg.val==74)				//Loopback Process

{

pm.vg.val=-1

//sys0 : Sender, sys1 : Command

sys2=pm.vv.val				//Loop back Value

pm.vv.val=0						//Init value for Next Receive process

sys0=sys2&0xFF				//Sender

//Loopback Signal

if(sys0>4)						//Sender < 5, ignore

{

sys2=sys2>>8

sys1=sys2&0xFF			//Command

if(sys1==10)				//Command : Backlight Off

{

sys2=sys2>>8			//

sys1=sys2&0xFF		//Data1

sys2=sys2>>8			//Data2 (Checksum

sys2=sys2&0xFF

sys0=sys0+sys1+10	//sender + command(10) + data1

sys0=sys0%256

if(sys0==sys2)		//Check Checksum

{

//Backlight Off

nSaverCount.val=-20	//Backlight off by Remote control

dim=0

}

}else if(sys1==11)		//Command : Backlight On

{

sys2=sys2>>8			//

sys1=sys2&0xFF		//Data1

sys2=sys2>>8			//Data2 (Checksum

sys2=sys2&0xFF

sys0=sys0+sys1+11	//sender + command(10) + data1

sys0=sys0%256

if(sys0==sys2)		//Check Checksum

{

//Backlight Off

dim=dims

nSaverCount.val=0

}

}

}

}

# CW Decode

## Preinitialize

pm.vv.val=-1

pm.vg.val=0

pm.sp.txt="J"

pm.sb.txt="^"

## Buttons

### bHome

TouchReleaseEvent

page px

### cSmeterToUart (checkbox to enable remote processor)

TouchReleaseEvent

randset 5,255

nMyAddr.val=rand

//Send

pm.vn.val=-1

pm.vx.val=0

printh 59								/MAGIC

printh 58								//MAGIC

printh 68								/MAGIC

prints 74,1								//Command : LOOP BACK

prints nMyAddr.val,1			//Loop Sender :

sys0=50+cSmeterToUart.val		//SMeterToUart

prints sys0,1				//Loop Command1 : Command Type / 95: Spectrum Mode

prints 0x6A,1				//Loop Command2 : Command Value / 0x6A to DSP

sys0=nMyAddr.val+sys0+0x6A

sys0=sys0%256

prints sys0,1				//Loop Command3 : Command Value / CheckSum

printh FF								//MAGIC

printh FF								//MAGIC

printh 73								//ETX

### b0 (Set limit)

TouchReleaseEvent

randset 5,255

nMyAddr.val=rand

//Send

pm.vv.val=-1

pm.vg.val=0

sys1=146+h0.val

printh 59										//MAGIC

printh 58										//MAGIC

printh 68										//MAGIC

prints 74,1							//Command : LOOP BACK

prints nMyAddr.val,1				//Loop Sender :

prints sys1,1			//Loop Command1 : Command Type / 100~129: Decode Morse

prints 0x6A,1			//Loop Command2 : Command Value / 0x6A to DSP

sys0=nMyAddr.val+sys1+0x6A

sys0=sys0%256

prints sys0,1			//Loop Command3 : Command Value / CheckSum

printh FF										//MAGIC

printh FF										//MAGIC

printh 73										//ETX

### b1 (Perhaps no opt?)

No events. 10 char max, defaults to empty string

### h0 (slider)

TouchReleaseEvent

randset 5,255

nMyAddr.val=rand

//Send

pm.vn.val=-1

pm.vx.val=0

printh 59										//MAGIC

printh 58										//MAGIC

printh 68										//MAGIC

prints 74,1							//Command : LOOP BACK

prints nMyAddr.val,1				//Loop Sender :

sys0=146+h0.val				//CW Limit

prints sys0,1					//Loop Command1 : Command Type

prints 0x6A,1					//Loop Command2 : Command Value / 0x6A to DSP

sys0=nMyAddr.val+sys0+0x6A

sys0=sys0%256

prints sys0,1					//Loop Command3 : Command Value / CheckSum

printh FF										//MAGIC

printh FF										//MAGIC

printh 73										//ETX

TouchMoveEvent

n1.val=h0.val*10

Max= 10, Min=0, Default=2

### nHigh

No events

Defined as Decimal

Default 0

### n0

No events

Defined as Decimal

Default 20

### nLow

No events

Defined as Decimal

Default 0

### nDecodeFreq

TouchPressEvent

sys0=hCWTone.val*50

nDecodeFreq.val=sys0+300

Default 800

### hCWTone (slider)

TouchMoveEvent

click nDecodeFreq,1

Default 10, min 0, max 45

### rFFT (radio button 1)

TouchPressEvent

if(rFFT.val==1)

{

rMorse.val=0

}

//else

//{

//  rMorse.val=1

//}

### rMorse (radio button 2)

TouchPressEvent

if(rMorse.val==1)

{

rFFT.val=0

}

//else

//{

//  rFFT.val=1

//}

### bSetCommand (apply)

TouchPressEvent

randset 5,255

nMyAddr.val=rand

//Send

pm.vv.val=-1

pm.vg.val=0

if(rFFT.val==1)	//Spctrum Mode

{

sys1=95

}else if(rMorse.val==1)

{		//Morse Decode

sys1=nDecodeFreq.val-300

sys1/=50

sys1+=100							//Decode Morse 100~129

}else

{

sys1=94

}

printh 59										//MAGIC

printh 58										//MAGIC

printh 68										//MAGIC

prints 74,1						//Command : LOOP BACK

prints nMyAddr.val,1				//Loop Sender :

prints sys1,1					//Loop Command1 : Command Type / 100~129: Decode Morse

prints 0x6A,1					//Loop Command2 : Command Value / 0x6A to DSP

sys0=nMyAddr.val+sys1+0x6A

sys0=sys0%256

prints sys0,1				//Loop Command3 : Command Value / CheckSum

printh FF										//MAGIC

printh FF										//MAGIC

printh 73

### tDecode1

TouchPressEvent

None

String length 30

### tDecode2

TouchPressEvent

None

String length 30

### tDecode3

TouchPressEvent

None

String length 30

## ***Timer 1***

if(pm.sp.txt!="J")

{

//fill 0,0,240,70,BLACK

//vMaxValue.val=0

//vMinValue.val=30000

//sys1=nDecodeFreq.val/48

sys1=nDecodeFreq.val/50

sys1=sys1+1

vSelectMin.val=sys1-1

vSelectMax.val=sys1+1

sys1=sys1*4

sys2=0

for(sys0=0;sys0<62;sys0++)

{

substr pm.sp.txt,sTemp0.txt,sys0*2,2

covx sTemp0.txt,sys2,2,2

sys2/=2

if(sys2<0)

{

sys2=0

}

if(sys2>70)

{

sys2=70

}

//if(vMaxValue.val<sys2)

//{

//  vMaxValue.val=sys2

//}

//if(vMinValue.val>sys2)

//{

//  vMinValue.val=sys2

//}

//sys1=sys0*2

//sys1+=194

//fill sys0*2,71-sys2,2,sys2,YELLOW		//1500+sys1*2046

//fill sys0*2,0,2,71-sys2,BLACK			//1500+sys1*2046

//fill sys0*4,240-sys2,4,sys2,YELLOW	//1500+sys1*2046

//fill sys0*4,170,4,70-sys2,BLACK			//1500+sys1*2046

fill sys0*4+9,110-sys2,4,sys2,YELLOW	//1500+sys1*2046

fill sys0*4+9,39,4,71-sys2,BLACK			//1500+sys1*2046

if(sys0>vSelectMin.val)

{

if(sys0<vSelectMax.val)

{

line sys1,40,sys1,109,RED

line sys1+4,40,sys1+4,109,RED

}

}

//if(sys1>nADCDisVal.val)					//maximum adc dccharget

//{

//  sys2=sys1-nADCDisVal.val

//}

//sys2=sys1*90	//80%

//sys2/=100

}

pm.sp.txt="J"

//Draw Decode Frequency

//sys1=nDecodeFreq.val/48

//sys1=sys1*4

//sys1+=4	//drop bin0 of FFT

//fill sys1,0,2,71,RED

//Graph LIne

//fill 9,110-n1.val,246,1,BLUE

//Needle

//line sys1,40,sys1,109,RED

//line sys1+4,40,sys1+4,109,RED

//Display High and Low value

//sys0=nHigh.val*7

//sys1=vMaxValue.val*3

//sys0+=sys1

//sys0/=10

//nHigh.val=sys0

//sys0=nLow.val*7

//sys1=vMinValue.val*3

//sys0+=sys1

//sys0/=10

//nLow.val=sys0

//fill 9,110-nHigh.val,246,1,GRAY

//nRecvCount.val--

//nArriveValue.val=0

//if(nRecvCount.val<1)

//{

//  click b1,1

//}

}else if(pm.sb.txt!="^")

{

if(vLineIndex.val==0)

{

tDecode1.txt=tDecode1.txt+pm.sb.txt

strlen tDecode1.txt,sys0

if(sys0>23)

{

vLineIndex.val=1

}

}else if(vLineIndex.val==1)

{

tDecode2.txt=tDecode2.txt+pm.sb.txt

strlen tDecode2.txt,sys0

if(sys0>23)

{

vLineIndex.val=2

}

}else if(vLineIndex.val==2)

{

strlen tDecode3.txt,sys0

if(sys0>23)

{

tDecode1.txt=tDecode2.txt

tDecode2.txt=tDecode3.txt

tDecode3.txt=""

}

tDecode3.txt=tDecode3.txt+pm.sb.txt

}

pm.sb.txt="^"

}else if(vReadStep.val==0)

{

vReadStep.val=1

pm.vv.val=-1

pm.vg.val=0

//Read Configuration from DSP

randset 5,255

nMyAddr.val=rand

printh 59										//MAGIC

printh 58										//MAGIC

printh 68										//MAGIC

prints 74,1					//Command : LOOP BACK

prints nMyAddr.val,1				//Loop Sender :

prints 2,1			//Loop Command1 : Command Type / 1~5: Read Configuration

prints 0x6A,1					//Loop Command2 : Command Value / 0x6A to DSP

sys0=nMyAddr.val+2+0x6A			//Byte0 + Byte1 + Byte2 % 256 = Checksum

sys0=sys0%256

prints sys0,1					//Loop Command3 : Command Value / CheckSum

printh FF										//MAGIC

printh FF										//MAGIC

printh 73										//ETX

}else if(vReadStep.val==1)

{

if(pm.vg.val==0x6A)					//Response from DSP

{

vReadStep.val=2

sys0=pm.vv.val&0xFF

sys1=pm.vv.val>>8

sys1=sys1&0xFF

sys2=pm.vv.val>>16

sys2=sys2&0xFF

//sys0 : magLimit

//sys1 : SMeterToUart

//sys2 : DSPType and Morse Decode HZ

if(sys0>100)

{

sys0=100

}

n1.val=sys0

sys0=sys0/10

h0.val=sys0

if(sys1==1)

{

cSmeterToUart.val=1

}else

{

cSmeterToUart.val=0

}

if(sys2==95)

{

rFFT.val=1

rMorse.val=0

}else if(sys2>=100)

{

if(sys2<146)

{

rFFT.val=0

rMorse.val=1

hCWTone.val=sys2-100

//nDecodeFreq.val=sys2

click nDecodeFreq,1

}

}

pm.vv.val=-1

pm.vg.val=0

}

}

### Variables

nMyAddr:		Number	0

sTemp0:		String		Max length 10

vReadStep:	Number	0

vSelectMin:	Number	0

vSelectMax:	Number	0

vLineIndex:	Number	0

# Spectrum

## Preinitialize

//Send Default Value

click bOption,1

nFreq.val=px.nLastFreq.val

nBeforeFreq.val=px.nLastFreq.val

vis bOption,0

## Buttons

### btnGetSpectrum

TouchPressEvent

if(nRecvCount.val<1)

{

if(nFreq.val>30000)

{

printh 59										//MAGIC

printh 58										//MAGIC

printh 68										//MAGIC

prints 16,1							//Command : Start Spectrum

sys0=nFreq.val

sys0=2000*nADCCount.val/2		//2k step

sys0=nFreq.val-sys0

print sys0									//Start Frequency

printh FF										//MAGIC

printh FF										//MAGIC

printh 73										//ETX

nRecvCount.val=nTargetCount.val

nStartFreq.val=sys0

fill 0,52,247,113,BLACK

}

}

### bHome

TouchReleaseEvent

page px

### bCancel

TouchReleaseEvent

//Set Frequency

printh 59										//MAGIC

printh 58										//MAGIC

printh 68										//MAGIC

prints 2,1									//Set Frequency

print nBeforeFreq.val				//VALUE

printh FF										//MAGIC

printh FF										//MAGIC

printh 73										//ETX

page px

### hViewFreq

TouchMoveEvent

nIsChangeFreq.val=1

Min:0, Max:245: Default=1

### h0 (slider)

TouchPressEvent

nPressValue.val=nFreq.val

TouchReleaseEvent

nPressValue.val=0

h0.val=500

TouchRMoveEvent

if(nPressValue.val>10000)

{

sys0=h0.val-500

sys0*=50

nFreq.val=nPressValue.val+sys0

}

Max:1000, Min:0 Default:500

### nFreq

No events - probably current frequency

### q0

No events - picc 15 -spacer from background color

### hCount

Touch Release Event

pm.vn.val=-1

pm.vx.val=0

//nStep.val=2000		//2Kh

nADCCount.val=120	//MAX : 120

printh 59										//MAGIC

printh 58										//MAGIC

printh 68										//MAGIC

prints 15,1							//Command : Spectrum Option

//printh 20									//Send Count

prints hCount.val,1					//Send Count (Scan Set Count)

printh 00			//ADC Value Offset (Spectrum Value = ADC Value - (Offset * 3)

prints nADCCount.val,1	//ADC Scan Count (120 - MAX Value)

sys0=2000/20

prints sys0,1					//Increase Step * 20 (ex 100 = 2Khz Step Scan)

printh FF										//MAGIC

printh FF										//MAGIC

printh 73										//ETX

//tm0.en=1

nTargetCount.val=hCount.val

Min: 2, Max:50, Default:10

nTargetCount

Look like just variable

### q1

No events - picc 15 -spacer from background color

### nRevCount

Look like just variable

## ***Timer tm0***

if(pm.sh.txt!="J")

{

fill 0,52,247,70,BLACK

for(sys0=0;sys0<120;sys0++)

{

substr pm.sh.txt,tmpS.txt,sys0*2,2

covx tmpS.txt,sys1,2,2

sys2=sys1-sys2

if(sys2<0)

{

sys2=0

}

if(sys2>70)

{

sys2=70

}

//Draw

fill sys0*2,122-sys2,2,sys2,YELLOW//1500+sys1*2046

if(sys2>40)

{

sys2=40

nTemp.val=RED

}else if(sys2>20)

{

nTemp.val=63910

}else if(sys2>15)

{

nTemp.val=64463

}else if(sys2>10)

{

nTemp.val=64940

}else if(sys2>8)

{

nTemp.val=YELLOW

}else if(sys2>3)

{

nTemp.val=48608

}else if(sys2>2)

{

nTemp.val=44352

}else

{

nTemp.val=25376

}

fill sys0*2,164-sys2,2,sys2,nTemp.val//1500+sys1*2046

//    if(sys1>nADCDisVal.val)	//maximum adc dccharget

//    {

//      sys2=sys1-nADCDisVal.val

//    }

if(sys1>7)		//maximum adc discharge

{

sys2=sys1-7

}

}

pm.sh.txt="J"

nRecvCount.val--

if(nRecvCount.val==0)

{

if(nSelectFreq.val>350000)

{

printh 59										//MAGIC

printh 58										//MAGIC

printh 68										//MAGIC

prints 2,1						//Set Frequency

print nSelectFreq.val				//VALUE

printh FF										//MAGIC

printh FF										//MAGIC

printh 73										//ETX

}

}

}

## ***Timer tmrDisplay***

sys2=0

if(nBeforeBase.val!=nFreq.val)

{

sys2+=1

}

if(nBeforePos.val!=hViewFreq.val)

{

sys2+=1

}

if(sys2>0)

{

nBeforeBase.val=nFreq.val

nBeforePos.val=hViewFreq.val

fill 0,0,247,52,10565

fill 0,25,247,2,16959

fill 120,10,2,15,16959

cov nFreq.val,tmpS.txt,0

xstr 126,4,90,18,0,WHITE,10565,0,1,1,tmpS.txt

sys0=nFreq.val

sys0=2000*nADCCount.val/2		//2k step fixed

sys0=nFreq.val-sys0

nStartFreq.val=sys0

cov nStartFreq.val,tmpS.txt,0

xstr 2,4,90,18,0,WHITE,10565,0,1,1,tmpS.txt

//fill 0,160,247,20,31727

fill 0,164,247,14,10565

fill 0,173,247,2,16959

sys0=hViewFreq.val

sys0=sys0*1000

sys1=sys0+nStartFreq.val

nSelectFreq.val=sys1

cov nSelectFreq.val,tmpS.txt,0

//xstr 150,26,90,24,2,64520,GRAY,2,1,1,tmpS.txt

if(hViewFreq.val>120)

{

xstr hViewFreq.val-92,28,90,24,2,64520,10565,2,1,1,tmpS.txt

}else

{

xstr hViewFreq.val+2,28,90,24,2,64520,10565,0,1,1,tmpS.txt

}

fill hViewFreq.val,0,2,50,64520

fill hViewFreq.val,164,2,14,64520

}

## ***Timer tmrSetFreq***

if(nIsChangeFreq.val==1)

{

if(nRecvCount.val==0)

{

if(nSelectFreq.val>350000)

{

if(nSelectFreq.val<57000000)

{

printh 59										//MAGIC

printh 58										//MAGIC

printh 68										//MAGIC

prints 2,1									//Set Frequency

print nSelectFreq.val				//VALUE

printh FF										//MAGIC

printh FF										//MAGIC

printh 73										//ETX

}

}

}

nIsChangeFreq.val=0

}

### Misc Variables

Tmp5: 			String		max 10

nPressValue: 		Number	0

nADCCount:		Number	0

nStartFreq:		Number	0

nSelectFreq:		Number	0

nBeforeFreq:		Number	0

nBeforePos:		Number	0

nBeforeBase:		Number	0

nBeforeBase:		Number	0

nIsChangeFreq:	Number	0