/*

FFT, CW Decode for uBITX

KD8CEC, Ian Lee

WC8C, Dennis Cabell

Version : 2

-----------------------------------------------------------------------

License : See fftfunctions.cpp for FFT and CW Decode.

**********************************************************************/

#include <Wire.h>

#include <EEPROM.h>

#include "i2cmeter2.h"

// #include "AltSoftSerial.h"

#include "CWDecoder.h"

#include "fftfunctions.h"

#include "NextionProtocol.h"

#include "PowerSwr.h"

#include "Configuration.h"

#ifdef USE_SW_SERIAL

AltSoftSerial SERIAL_OUTPUT;

NextionProtocol nextion = NextionProtocol(SERIAL_OUTPUT);

#else

NextionProtocol nextion;

#endif

#ifdef ENABLE_POWER_SWR_METER

PowerSwr powerSwrCalculator;

#endif

CWDecoder decoder;

int FFTReal[SAMPLESIZE];

int ADC_DIFF  = 0;

volatile byte I2CCommand = 0; // volatile because I2C interrupt changes this

int GrepADC(int readSampleCount, int *readArray)

{

#ifdef USE_SW_SERIAL

// if there is data to still output then return so normal forwarding can continue.

// only want to sample if the SW Serial output is empty so extra interrupts dont disrupt sample timing.

if (!SERIAL_OUTPUT.TxBufferIsEmpty())

{

return -1;

}

#endif

unsigned long currentms = 0;

unsigned long newCurrentms = 0;

int ADC_MAX = 0;

int ADC_MIN = 30000;

analogRead(SIGNAL_METER_ADC);  // initial unused value helps accuracy since ADC may have been switched to read power & swr.

newCurrentms = micros();

for(int i=0; i < readSampleCount; i++)

{

if (nextion.SerialDataToProcess())

{

// if something has come in then terminate the FFT sample so incomming data can be processed.

// this is so that frequency tuning does no lag when in FFT mode.

// returning -1 instead of ADC_DIFF indicates that the sample is incomplete.

return -1;

}

currentms = newCurrentms;

readArray[i] = analogRead(SIGNAL_METER_ADC);

ADC_MAX = max(ADC_MAX, readArray[i]);

ADC_MIN = min(ADC_MIN, readArray[i]);

while((newCurrentms - currentms) < SAMPLE_INTERVAL){newCurrentms = micros();}

} //end of for

return ((ADC_MAX - ADC_MIN) * SMETER_RESOLUTION_MULTIPLIER) / SMETER_GAIN;  // return ADC_DIFF

}

void setup()

{

Wire.begin(I2CMETER_ADDR);              //j : S-Meter Slave Address

Wire.onReceive(I2CReceiveEvent);

Wire.onRequest(I2CRequestEvent);

Serial.begin(SERIAL_SPEED, SERIAL_8N1);

Serial.flush();

#ifdef USE_SW_SERIAL

SERIAL_OUTPUT.begin(SERIAL_SPEED);

#endif

nextion.Initialize();

//spend time just forwaring data while everything starts up.  There is a lot of initial data to transfer.

unsigned long startMillis = millis();

while (millis() - startMillis < 2000)

{

nextion.ForwardData();

}

}

void loop()

{

while (nextion.SerialDataToProcess())

{

// This will continue to loop until and enture command is forwarded.

// Only add things here if it must absolutely inlterleave with forwarding a command from the uBitx.

// Note that the I2C Request work cannot be moved here because Wire.Write must stay in the ISR to function correctly.

nextion.ForwardData();  // this will not return until the input UART is empty

}

//===========================================

//Processing for when a config response has needs to be sent

//===========================================

if (nextion.ResponseConfig())

{

return; // if anything was sent to nextion, then start loop over so any data that may have come in can be processed.

}

//===========================================

//Processing for when the frequency has changed

//===========================================

if (nextion.frequencyHasChanged)

{

// TBD: Call process to do when the frequency has changed (e.x. output to an antenna tuner)

nextion.frequencyHasChanged = false;

return; //Start a new loop so more serial data can be processed

}

//===========================================

//TRANSCEIVER STATUS : TX

//===========================================

#ifdef ENABLE_POWER_SWR_METER

static uint8_t pwrSwrResetCounter = 0; // when TX disabled, how many times to try to reset the power and SWR to ensure Nextion gets it

#endif

if (nextion.TXStatus == 1 || !nextion.nextionIsConnected)  //  TX Mode, or always read if there is no nextion so values are available if requested via I2C

{

#ifdef ENABLE_POWER_SWR_METER

powerSwrCalculator.PowerSwrCalculation(nextion.L_vfoCurr);

if (nextion.nextionIsConnected)

{

nextion.SendPowerSwr(powerSwrCalculator.power, powerSwrCalculator.swr, true);

pwrSwrResetCounter = POWER_SWR_RESET_RETRY_COUNT;

return; //Do not process ADC, FFT, Decode Morse or RTTY, only Power and SWR Data Return

}

#endif

}

else

{

#ifdef ENABLE_POWER_SWR_METER

if (nextion.nextionIsConnected && pwrSwrResetCounter > 0)

{

//  reset the power and SWR meter when state changes out of TX mode

if (!nextion.SendPowerSwr(0.0, 1.0, false))

{

return;  // if incomming data aborted the send, return so it can be processed

}

pwrSwrResetCounter--;

}

#endif

}

//===========================================

//TRANSCEIVER STATUS : RX or no Nextion

//===========================================

//===========================================

// ADC Sampling

//===========================================

int new_ADC_DIFF = GrepADC((nextion.DSPType == 2 || nextion.DSPType == 3) ? DECODE_MORSE_SAMPLESIZE : SAMPLESIZE, FFTReal);

if (new_ADC_DIFF < 0)

{

return; // If the sample was not completed, then return so incomming data can be processed in the next loop.

}

noInterrupts();         // disable interrupts while saving these so that the I2C ISR cant read them while they are changing.

ADC_DIFF = new_ADC_DIFF;

interrupts();

nextion.CalculateScaledSMeter(ADC_DIFF);

//===================================================================================

// DSP Routine - only if there is a Nextion

//===================================================================================

if (nextion.nextionIsConnected)

{

if (nextion.DSPType == 0)

{

nextion.SendScaledSMeter(FastMeterToUartInterval);

}

else if (nextion.DSPType == 1 && nextion.LastSendWaitTimeElapsed()) //Spectrum : FFT => Send To UART

{

nextion.SendScaledSMeter(FFTMeterToUartInterval);

nextion.SendFFTData(SAMPLESIZE, FFTReal); // if new data comes in, the send gets aborted and control returns so the loop can be started over.

}

else if (nextion.DSPType == 2)          //Decode Morse

{

decoder.Decode_Morse(GetMagnatude(nextion.coeff, DECODE_MORSE_SAMPLESIZE, FFTReal), nextion.magnitudelimit_low);

if (!nextion.SerialDataToProcess())  // only send decoded data if nothing new has come in to process.

{

if (nextion.SendDecodeCharacters(decoder.GetDecodedCharacters()))

{

decoder.ClearDecodedCharacters();  // if send succeeded clear the characters

}

nextion.SendScaledSMeter(DecodeMeterToUartInterval);// Note that if characters were sent then this will defer until LAST_TIME_INTERVAL has elapsed.  They wont send back to back

}

}

}

} //end of main loop

void I2CReceiveEvent(int)

// void I2CReceiveEvent(void)

{

int readCommand = 0; // byte를 읽어 int로 변환

while(Wire.available() > 0)     // for Last command

{

readCommand = Wire.read();

}

if (0x50 <= readCommand && readCommand <= 0x59)

{

I2CCommand = readCommand;

}

}

void I2CRequestEvent(void)

{

unsigned long startMicros = micros();

byte command = I2CCommand; // make a copy in case it case the other interrupt changes it while this is executing

if (command == I2CMETER_CALCS)

{

Wire.write(nextion.scaledSMeter/SMETER_RESOLUTION_MULTIPLIER);  // only sends low resolution SMeter

}

else if (command == I2CMETER_UNCALCS)

{

int maxValue = 0;

int minValue = 30000;

int adcValue = 0;

while((micros() - startMicros) < 900)  // read as many as we can in 1 alotted time.  Longer can cause issues with the timers

{

adcValue = analogRead(SIGNAL_METER_ADC);;

maxValue = max(adcValue, maxValue);

minValue = min(adcValue, minValue);

}

// cast to a long becasue this theoretically can be 1023 * 1023, then cast it back down after the constrain

// Not sure why it is being squared - maintianing behavior of original code.

long sampleDifference = (float)(maxValue - minValue) / SMETER_GAIN;

Wire.write((uint8_t)constrain(sampleDifference * sampleDifference, 0, 255));

}

/* Notes on power and SWR

*

* Original code sends an int, so this does too, but this will get converted to 8 bits if the value is > 255

* constraining to 255 seems too small. Ask Dr. Lee

*

* Dont need to read if in RX mode, just send zero because when not transmitting the value should always read zero

* however if there is no nextion connected (i.e. there is no serial input), then the TX status is unknown

* so the data needs sent.

*

* Note that the analog reads happen in the main loop and the values are saved so they are available here.

* This is to keep the interrupt fast because the analogRead is a bit slow.

*/

else if (command == I2CMETER_CALCP)    // POWER

{

int value = 0;

#ifdef ENABLE_POWER_SWR_METER

if (nextion.TXStatus == 1 || !nextion.nextionIsConnected)  //TX Mode or there isn't a nextion

{

value = powerSwrCalculator.powerIn;

}

#endif

Wire.write(value);  //POWER

}

else if (command == I2CMETER_CALCR)        // REV POWER

{

int value = 0;

#ifdef ENABLE_POWER_SWR_METER

if (nextion.TXStatus == 1 || !nextion.nextionIsConnected)  //TX Mode or there isn't a nextion

{

value = powerSwrCalculator.revPowerIn;

}

#endif

Wire.write(value);  //Reverse Power

}

}